Assignment 3

16-bit ALU

Introduction: -

ALU is the fundamental building block of the processor, which is responsible for carrying out the arithmetic, logic functions, Shift functions and Comparison functions.

Specification:

- ALU Operands (A, B)

- ALU Result (ALU_OUT)

- ALU operands and output Result are of 16-bit width.

- ALU Result (ALU_OUT) is registered.

- The ALU function is carried out according to the value of the ALU_FUN input signal stated in the table in the following page and any other value for ALU_FUN not stated in the table, ALU_OUT must equal to 16’b0

- Arith_flag is activated "High" only when ALU performs one of the arithmetic operations (Addition, Subtraction, Multiplication, division), otherwise "LOW"

- Logic_flag is activated "High" only when ALU performs one of the Boolean operations (AND, OR, NAND, NOR, XOR, XNOR), otherwise "LOW"

- CMP_flag is activated "High" only when ALU performs one of the Comparison operations (Equal, Greater than, less than), otherwise "LOW"

- Shift_flag is activated "High" only when ALU performs one of the shifting operations (shift right, shift left), otherwise "LOW"

ALU_FUN Table

| ALU_OUT | Operation | ALU_FUN |
| --- | --- | --- |
|  | Arithmatic : Addition | 0000 |
|  | Arithmatic : Subtraction | 0001 |
|  | Arithmatic : Multiplication | 0010 |
|  | Arithmatic : Division | 0011 |
|  | Logic : AND | 0100 |
|  | Logic : OR | 0101 |
|  | Logic : NAND | 0110 |
|  | Logic : NOR | 0111 |
|  | Logic : XOR | 1000 |
|  | Logic : XNOR | 1001 |
| Equal to 1 | CMP: A = B | 1010 |
| Equal to 2 | CMP: A > B | 1011 |
| Equal to 3 | CMP: A < B | 1100 |
|  | SHIFT: A >> 1 | 1101 |
|  | SHIFT: A << 1 | 1110 |

Hint: Use Case statement to describe the behavior of this table and use default case if needed.

Hint: You can use if statement inside case branches

Block Interface

- Write a Verilog Code to capture the above specifications as well as the synthesis diagram of your code.

- Write a testbench to test all the ALU functions with operating clock frequency 100 KHz