# Signals and Systems 1

- Time scaling xt→xat divides the time axis by a.

- When more than one transformation is combined (e.g. x-2t-1, the easiest order is:

  - Time shift

  - Time scale

  - Time reverse

- Signal types according to power and energy:

- All periodic signals are power signals.

- For an odd signal, x0=0.

- Any signal xt=xet+xot, where:

  - xet=xt+x-t2

  - xot=xt-x-t2

- A unit step signal is neither even nor odd, and it’s a power signal.

- A unit impulse signal is even, and it’s neither energy nor power signal.

- Relationship between unit step and unit impulse:

  - ut=-∞tδτdτ

  - δt=dutdt

- What is a memoryless system?

  - A system is memoryless if the output at a given time depends only on the input at the same time but not in past times.

- What is a causal system?

  - A system is causal if the output at a given time depends only on the input at the present and past times but not future.

- What is an invertible system?

  - A system is invertible if we can figure out an inverse system for it:

  - For this to be possible, the system must generate distinct outputs for distinct inputs; if the system can generate the same output for two different inputs, then it is non-invertible.

  - To prove that a system is non-invertible, show that it gives the same output for two different inputs.

  - To prove that a system is invertible, try to get xt (i.e. the system’s input) as a function of yt.

- What is a linear system?

  - Linear = additive + homogenous

  - Additive: if x1t gives y1t and x2t gives y2t, then x1t+x2t should give y1t+y2t if the system is additive.

  - Homogenous: if x1t gives y1t, then ax1t should give ay1t if the system is homogenous.

  - Together: if x1t gives y1t and x2t gives y2t, then a1x1t+a2x2t should give a1y1t+a2y2t if the system is linear.

- What is a time-invariant system?

  - If a system is time-invariant, then a time shift in the input should result in an identical time shift in the output. That is, if xt gives yt, then xt-to would give yt-to if the system is time-invariant.

- Is an integrator system causal/non-causal and memoryless/with memory?

  - An integrator system is given by yt=-∞txτdτ, so it’s with memory and causal.

- Is yt=x-t causal or non-causal?

  - Non-causal (try to put negative time values)

- Is yt=sinxt an invertible system?

  - If xt=0, then yt=0. If xt=π, then yt=0. Thus, non-invertible.

- Is yt=x2t a time-invariant system?

- If the input to a time-invariant system is periodic, the output will also be periodic with the same period.

- An LTI system is special because it is completely characterized by its impulse response h, which determines the output for any input via convolution in time or multiplication in frequency.

- For an LTI system:

  - yt=xt*ht=-∞∞xτht-τdτ

  - The start of yt is the sum of starts of xt and ht, and the end of yt is the sum of xt and ht.

- xtδt-to=xtoδt-to

- xt*δt-to=xt-to

- LTI system properties from its impulse response ht:

  - An LTI system is said to be memoryless if ht=kδt.

  - An LTI system is said to be causal if ht=0 for t<0.

  - An LTI system is said to be invertible if we can find hit such that ht*hit=δt OR HωHiω=1

# Signals and Systems 2

- Discrete-time shabah bzbt el continuous-time, fa I will mention bs el 7agat el momken fiha e5telaf aw tafkeer.

- t→n

- ω→Ω

- Is the system yn=m=-∞nxm invertible?

  - yn=yn-1+xn→xn=yn-yn-1

  - Always remember that the accumulator and difference systems are the inverse of each other (just like differentiator and integrator systems).

- If a sequence of length N1 is convolved with a sequence of length N2, then the output sequence is of length N1+N2-1.

# Internship – Session 1

- Analog modulation: it is the process of varying an analog carrier signal (amplitude, phase, or frequency) in accordance with a message signal.

- Digital modulation: it is the process of mapping digital data (i.e. bits) onto analog waveforms by altering a carrier signal’s amplitude, phase, or frequency.

- To be able to reconstruct any signal (whether baseband or passband):

fs≥2×BW

- The PSD of a staircase signal is ∝sinc2πfTsymbol. Thus, all modulation schemes result in the same PSD:

- We have two independent quantities: Energy and Rate:

  - Energy:

    - It can be symbol energy Es or bit energy Eb, where Eb=Esn and n=log2M.

    - What decides energy is the symbol constellation. Taking M-PSK as an example, for the same modulation order, the farther the symbols are from each other (i.e. the wider the circle is), the higher the symbol energy and accordingly the bit energy. Or, the higher the modulation order for the same circle (which means same symbol energy), the lower the bit energy is.

  - Rate:

    - It can be symbol rate Rs or bit rate Rb, where Tsymbol=nTbit→Rs=Rbn and n=log2M.

    - Rs=fclk of the digital domain, so what decides the rate is the rate by which the digital domain transmits symbols to the DAC.

    - Symbol rate decides the bandwidth as BW≈1Tsymbol=Rs.

# Internship – Session 2

- DFT:

- L defines how accurate the spectrum is. N defines how smooth it looks.

- Going from single-carrier to FDM keeps the same data rate but makes the system more robust to ISI thanks to having each subcarrier seeing a smaller and hence flatter channel.

- Going from FDM to OFDM:

  - increases spectral efficiency since we pack more subcarriers in the same bandwidth.

  - results in higher data rate since we’re transmitting more symbols in the same symbol duration.

  - Makes the system more robust since each subcarrier became narrower and see flatter channel.

  - Has no negative impact on the errors since all subcarriers are orthogonal.

# Internship – Session 3

- Slides 10 to 19.

# DPD

- A classical adaptive FIR filter is given by:

yn=i=0L-1hixn-i

where the coefficients are updated over time (i.e. not constant) using adaptive algorithms (e.g. LS, LMS, NLMS, RLS).

- The DPD is the most sophisticated and advanced adaptive filtering system used in modern RF transmitters. Why? Because:

  - Its model is non-linear, not linear as the simple adaptive FIR filters.

  - The feedback path needed to adaptively update the coefficients introduces distortions.

  - To account for adjacent channels, a DPD system requires 2-4x oversampling which puts a burden on the digital implementation side.

- The Q matrix in QR is a unitary matrix (i.e. has a unity norm).

- Givens rotation rotates a vector AB clockwise by an angle θ while preserving the vector’s length (this is why it is called a unitary transformation) by multiplying by a unitary matrix:

cosθsinθ-sinθcosθAB=EF

- To transform a 3x3 real matrix into an upper triangular matrix:

A11A12A13A21A22A23A31A32A33

Ψ1A11A12A13A21A22A23A31A32A33=A11'A12'A13'0A22'A23'A31A32A33

Ψ2Ψ1A11A12A13A21A22A23A31A32A33=A11''A12''A13''0A22'A23'0A32'A33'

Ψ3Ψ2Ψ1A11A12A13A21A22A23A31A32A33=A11''A12''A13''0A22''A23''00A33''

Ψ3Ψ2Ψ1A=R

A=Ψ3Ψ2Ψ1TR

Q=Ψ3Ψ2Ψ1T

- To transform a 3x3 complex matrix into an upper triangular matrix:

U6U5U4U3U2U1A=R

A=U6U5U4U3U2U1-1R

A=U1HU2HU3HU4HU5HU6HR

Q=U1HU2HU3HU4HU5HU6H