Assignment 4.1

16-bit ALU

Introduction: -

ALU_TOP is the fundamental building block of the processor, which is responsible for carrying out different functions: -

- Arithmetic functions through ARITHMETIC_UNIT block.

- Logic functions through LOGIC_UNIT block.

- Shift functions through SHIFT _UNIT block.

- Comparison functions through CMP_UNIT block.

And Decoder Unit responsibles for enable which Function to operate according to the highest Most significant 2-bit of the ALU_FUNC control bus ALU_FUNC [3:2].

Block Diagram

TOP Module (ALU_TOP) Port Description:

| Width (bits) | Signal Name |
| --- | --- |
| parameterized | A |
| parameterized | B |
| 4 | ALU_FUNC |
| 1 | CLK |
| parameterized | Arith_OUT |
| 1 | Carry_OUT |
| 1 | Arith_Flag |
| parameterized | Logic_OUT |
| 1 | Logic_Flag |
| parameterized | CMP_OUT |
| 1 | CMP_Flag |
| parameterized | SHIFT_OUT |
| 1 | SHIFT_Flag |

Specifications: -

- All Outputs are registered.

- Arith_flag is activated "High" only when ALU performs one of the arithmetic operations (Addition, Subtraction, Multiplication, division), otherwise "LOW"

- Logic_flag is activated "High" only when ALU performs one of the Boolean operations (AND, OR, NAND, NOR), otherwise "LOW"

- CMP_flag is activated "High" only when ALU performs one of the Comparison operations (Equal, Greater than, less than) or NOP, otherwise "LOW"

- Shift_flag is activated "High" only when ALU performs one of the shifting operations (shift right, shift left), otherwise "LOW"

- The ALU function is carried out according to the value of the ALU_FUN input signal stated in the following table.

ALU_FUN Table

| ALU_OUT | Operation | ALU_FUN |
| --- | --- | --- |
|  | Arithmatic : Addition | 0000 |
|  | Arithmatic : Subtraction | 0001 |
|  | Arithmatic : Multiplication | 0010 |
|  | Arithmatic : Division | 0011 |
|  | Logic : AND | 0100 |
|  | Logic : OR | 0101 |
|  | Logic : NAND | 0110 |
|  | Logic : NOR | 0111 |
| Equal to 0 | NOP | 1000 |
| Equal to 1 | CMP: A = B | 1001 |
| Equal to 2 | CMP: A > B | 1010 |
| Equal to 3 | CMP: A < B | 1011 |
|  | SHIFT: A >> 1 | 1100 |
|  | SHIFT: A << 1 | 1101 |
|  | SHIFT: B >> 1 | 1110 |
|  | SHIFT: B << 1 | 1111 |

Hint: Use Case statement to describe the behavior of this table and use default case if needed.

Hint: You can use if statement inside case branches.

Note: Arith_Enable, Logic_Enable, SHIFT_Enable and CMP_Enable are called block enable which responsible for enabling the function of the block or not.

Decoder Truth Table

| SHIFT_EN | CMP_EN | Logic_En | Arith_En | ALU_FUNC[3:2] |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 00 |
| 0 | 0 | 1 | 0 | 01 |
| 0 | 1 | 0 | 0 | 10 |
| 1 | 0 | 0 | 0 | 11 |

Hint:  How to use the enable signal inside the code of each block.

always @(*)

begin

if(Arith_Enable)

begin

case(ALU_FUN)

2’b00 : {ALU_Carry, ALU_Arith } = A + B ;

………..

endcase

end

else

begin

ALU_Arith = 16’b0 ;

End

End

Requirements: -

- Write a Verilog Codes of the following 6 modules

  - ARITHMETIC_UNIT

  - LOGIC_UNIT.

  - SHIFT _UNIT

  - CMP_UNIT

  - Decoder Unit

  - ALU_TOP

- Write a testbench to test all the ALU functions with operating clock frequency 100 KHz with duty cycle 40% low and 60% high