# Digital Logic Families

## Bipolar Logic Family

### Resistor-Transistor Logic (RTL)

## MOS Logic Family

### Resistor-Transistor Logic (RTL)

### NMOS Enhancement Load Logic Family

### CMOS Logic Family

## Bipolar Logic Family

### Resistor-Transistor Logic (RTL)

- 3ayzeen nemla el table da 3ashan ne decide whether the bipolar logic family is good or bad.

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' |  |  |  |  |
| '1' |  |  |  |  |

3ashan neda5al logic ‘0’ w logic ‘1’ handa5al voltage kam? Akeed 0 V w Vcc.

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V |  |  |  |
| '1' | Vcc |  |  |  |

Logic ‘0’ Case:

3ayzeen ne3raf el region of operation (Q) bta3t el BJT. Da5el 3al base 0 V yeb2a akeed off. Sa3et-ha Vout hateb2a bkam? Be Vcc, which is the perfect logic ‘1’.

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | Vcc | Cut-off |  |
| '1' | Vcc |  |  |  |

El current 2ad eh? open circuit yeb2a zero.

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | Vcc | Cut-off | 0 |
| '1' | Vcc |  |  |  |

Logic ‘1’ Case:

3ayzeen ne3raf el region of operation (Q) bta3t el BJT. Da5el 3leeh Vcc fa howa akeed on. Tyb ya tara active wala saturation? De b2a enta el te7adedha ka designer, enta el te design el area bta3t el BJT wel resistor el fel circuit such that enak te5aly el transistor fel region of operation el enta 3ayezha. Tab e7na 3ayzeno yeb2a eh? e7na 3ayzeen el output yeb2a logic ‘0’, y3ny voltage olayel, yeb2a ana 3ayez Vout a2al ma yomken. Hal yenfa3 teb2a be zero? La2 la2en a2al voltage 3la el BJT beyeb2a fel saturation region w beyeb2a be VCE,sat. Yeb2a keda a2al value le Vout heya VCE,sat wel region of operation heya saturation. Tab wel current? Hayeb2a be Isat.

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | Vcc | Cut-off | 0 |
| '1' | Vcc | VCE,sat | Saturation | Isat |

3ayzeen bs n3araf 2 quantities:

- VOL: The best possible logic ‘0’ voltage that a family can output.

- VOH: The best possible logic ‘1’ voltage that a family can output.

Tab3an hayeb2a dayman nefsena en VOL teb2a be 0 V w VOH teb2a be VDD. Tab hena VOL & VOH equal kam?

VOL=VCE,sat

VOH=Vcc

Yeb2a eh el disadvantages beta3t el BJT family el wad7a fel table el faat?

      - VOL is not 0 V but VCE,sat.

      - In case of logic ‘1’ input, there is a static current Isat flowing.

Actually, fi ghalta e7na 3amlenha fel truth table keda. Eh heya? Enena ektashafna en el logic ‘0’ output howa mesh 0 V la2 howa VCE,sat. Tab eh el ghalat f keda? El ghalat f keda enena fel case beta3t logic ‘0’ input e3tabarna en el voltage be 0 V, fa da ghalat la2en e7na bene3teber en el hayedeny el input beta3y howa gate zayee bzbt, wana lesa ba2ol en ana a7san logic ‘0’ hatala3o howa VCE,sat, fa menen haygely el 0 V da? W bel taly lazem ne modify el table as follows:

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | VCE,sat | Vcc | Cut-off | 0 |
| '1' | Vcc | VCE,sat | Saturation | Isat |

Mafeesh 7aga tanya hatetghayar fel table? As long as VCE,sat is less than VBE,ON(≈0.7V) (w da 7a2e2y f3ln), the BJT will remain in cut-off and hence nothing will change. Fa el e7na dayman hane3melo fi ay family enena nebda2 be en el logic ‘0’ be 0 V, w nshoof haytala3lena logic ‘1’ voltage bkam, el hayetla3 yeb2a howa el VOH, na5od el VOH da neda5alo tany ka input w el yetla3 howa VOL, walahy tele3 0 V 5eer w baraka, tele3 7aga tanya nerga3 n3adel el row beta3 el logic ‘0’ input fel table zy ma 3amalna hena keda.

Now let us cascade some BJTs after that BJT (called fanout):

El table hayetghayar ezay fel case de?

Fel case beta3t logic ‘1’ input mafeesh 7aga hatetghayar. Fel case beta3t logic ‘0’ input, fa el BJT hayeb2a open-circuit bardo tamam. However, hal el current el hayeb2a mashy hayeb2a be zero? La2, la2en el BJTs el tanya betes7ab base current, w bel taly el voltage 3and el output beta3 awel BJT (called driver) mesh hayeb2a Vcc, w enama hayeb2a Vcc-nIb*RL. Thus:

| Logic Input | Vin | Vout | Q | Current |
| --- | --- | --- | --- | --- |
| '0' | VCE,sat | Vcc-nIb*RL | Cut-off | 0 |
| '1' | Vcc | VCE,sat | Saturation | Isat |

Yeb2a keda el disadvantages beta3t el BJT logic family el mesh hat5alena nesta5demha heya:

- The logic ‘0’ output is not a perfect logic ‘0’ but VCE,sat.

- In case of logic ‘1’ input, there is a static current Isat flowing.

- In case of logic ‘0’ input, the output depends on the fanout (degrades as the fanout increases).

Another problem heya en el BJT fabrication beyeb2a as shown below:

Eh el moshkela f keda? El moshkela en el substrate heya el collector terminal, fa lw gena ne fabricate 2 BJTs gamb b3d hayeb2a lehom common collector as shown below, fi 7een eny makontesh 3ayez awasal el collectors beto3hom wala 7aga.

3ashan ne7el el moshkela de ben7ot p-type fel nos mabenhom w nwasalha bel ground, b7ees keda yeb2a el 2 PN junctions yemen w shmaal reverse biased as shown below.

Fa tab3an da yo3tabar waste of area en mabeen kol 2 BJTs aroo7 7atet p-type.

## MOS Logic Family

### Resistor-Transistor Logic (RTL)

| Logic Input | Vin | Vout | M | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V |  |  |  |
| '1' | VDD |  |  |  |

Logic ‘0’ Case:

Logic ‘0’ y3ny el MOSFET cut-off, w bel taly kol 7aga et7adedet:

| Logic Input | Vin | Vout | M | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | VDD | Cut-off | 0 |
| '1' | VDD |  |  |  |

Logic ‘1’ Case:

Vin=VDD yeb2a mabda2eyan el transistor on. Sat wala triode? As2al nafsy ana 3ayzo eh? walahy ana nefsy Vout teb2a be 0 V, y3ny 3ayez el voltage drop 3al transistor Asghar ma yomken, y3ny 3ayez el resistance beta3to teb2a soghayara gedan. Anhy region beteb2a fiha resistance el MOSFET soghayara? El deep triode (ohmic) region. Thus, ha assume eno fel ohmic region w ab2a a verify this assumption later.

| Logic Input | Vin | Vout | M | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | VDD | Cut-off | 0 |
| '1' | VDD |  | ohmic |  |

Hane7seb VOL w I ezay? Dayman et3awed enena hane3mel KCL at the output:

IR=IMOSFET

VDD-VOLR=KVDD-VTHVOL-VOL22

Then han solve this quadratic equation, hayetla3lena 2 solutions, bs solution wa7ed bs fihom howa el ye5aly el MOSFET fel ohmic region (w keda teb2a bel mara verified your assumption). Fadel ne7seb el current, simply hana5od VOL el 7asabnaha de w n7otaha fi equation IR aw IMOSFET neb2a 7asabna el current.

| Logic Input | Vin | Vout | M | Current |
| --- | --- | --- | --- | --- |
| '0' | 0 V | VDD | Cut-off | 0 |
| '1' | VDD | VOL | ohmic | Iohmic |

Again, VOL matel3etsh 0 V zy ma kona moftaredeen. Yeb2a lazem ne modify el table in case of logic ‘0’ input. Fi ay 7aga tanya hatetghayar? As long as VOL is smaller than VTH (w heya f3ln hateb2a smaller), mafeesh 7aga hatetghayar.

| Logic Input | Vin | Vout | M | Current |
| --- | --- | --- | --- | --- |
| '0' | VOL | VDD | Cut-off | 0 |
| '1' | VDD | VOL | ohmic | Iohmic |

Disadvantages:

- VOL is not 0 V.

- In case of logic ‘1’ input, there is a static current Iohmic flowing.

Yeb2a e7na moshkeletna ma3 el family de heya el VOL, nefsena nesagharha 3la 2ad ma ne2dar. 3ashan ne2alel VOL, el mafrood n3ml eh fi WL w R?

3ashan negaweb el so2al da, me7tag tefakar fel circuit el foo2 enaha voltage divider mabeen el R wel RMOSFET, w en el voltage el 3la el MOSFET howa VOL de. Enta 3ayez te2alel VOL, yeb2a akeed hankabar R b7ees nekabar el voltage drop 3leeha w bel taly VOL te2el. Tyb WL? We know that

RMOSFET∝1I∝1WL

W bema enena 3ayzeen RMOSFET tesghar, yeb2a 3ayzeen WL tezeed. Thus, to decrease VOL:

- R ↑

- WL ↑

However, we know that resistors are hard to integrate on chip, so we move forward to the next logic family.

### NMOS Enhancement Load Logic Family

| Logic Input | Vin | Vout | MD | ML | Current |
| --- | --- | --- | --- | --- | --- |
| '0' |  |  |  |  |  |
| '1' |  |  |  |  |  |

Logic Input ‘0’ Case:

Vin=0 V

MD→off, I=0

ML→Sat (diode-connected)

Hane7seb Vout ezay? Again, KCL at the output:

IML​sat=IMD​off

KL2VDD-VOH-VTHL2=0→VOH=VDD-VTHL

Notice that VTHL is large due to body effect.

| Logic Input | Vin | Vout | MD | ML | Current |
| --- | --- | --- | --- | --- | --- |
| '0' | 0 V | VDD-VTHL | Off | Sat | 0 |
| '1' |  |  |  |  |  |

Logic Input ‘1’ Case:

Vin=VDD-VTHL

ML→Sat.

MD→ ?

Again, by design, ana 3ayez Vout delwa2ty yeb2a low voltage 3ashan da inverter, w bel taly 3ayez MD yeb2a el voltage drop 3leeh olayel, yeb2a 3ayzo yeb2a fel ohmic region. Fa han assume eno ohmic w neb2a nerga3 ne verify fel a5er.

Hane7seb VOL w I ezay? Again, KCL at the output:

ID​ohmic=IL​sat

KDVDD-VTHL-VTHDVOL-VOL22=KL2VDD-VOL-VTHL2

Solve for VOL and verify your assumption (i.e.  VOL<VDD-VTHL-VTHD)

get I

| Logic Input | Vin | Vout | MD | ML | Current |
| --- | --- | --- | --- | --- | --- |
| '0' | 0 V | VDD-VTHL | Off | Sat | 0 |
| '1' | VDD-VTHL | VOL | Ohmic | Sat | I |

Disadvantages of this logic family:

- VOH= VDD-VTHL not VDD.

- VOL is not 0 V.

- In case of logic ‘1’ input, there is a static current flowing.

What is the power dissipation in this inverter?

Power Dissipation=

Power Dissipation In case of Logic '1'+Power Dissipation In case of Logic '0'2=VDD*IOL+02=VDD*IOL2

## NOR Gate

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |
| 0 | 1 |  |  |  |  |  |
| 1 | 0 |  |  |  |  |  |
| 1 | 1 |  |  |  |  |  |

‘0’ ‘0’: easy (hane3mel KCL at the output kal3ada, ma3andenash equation gheer de).

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Cut-off | Cut-off | Sat. | 0 | VDD-VTHL |
| 0 | 1 |  |  |  |  |  |
| 1 | 0 |  |  |  |  |  |
| 1 | 1 |  |  |  |  |  |

‘0’ ‘1’: M1 cut-off 3ady, M2? By design, e7na 3ayzeen yeb2a el output low voltage, w bel taly lazem M2 yeb2a ohmic.

Current and Vout? Again, KCL at the output:

IL=IM2

KL2VDD-VOL1-VTHL2=KM2VDD-VTHL-VTHM2VOL1-VOL122

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Cut-off | Cut-off | Sat. | 0 | VDD-VTHL |
| 0 | 1 | Cut-off | Ohmic | Sat. | IOL1 | VOL1 |
| 1 | 0 |  |  |  |  |  |
| 1 | 1 |  |  |  |  |  |

‘1’ ‘0’: Nafs el case el fatet, M1 w M2 bs badelo:

IL=IM1

KL2VDD-VOL2-VTHL2=KM1VDD-VTHL-VTHM1VOL2-VOL222

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Cut-off | Cut-off | Sat. | 0 | VDD-VTHL |
| 0 | 1 | Cut-off | Ohmic | Sat. | IOL1 | VOL1 |
| 1 | 0 | Ohmic | Cut-off | Sat. | IOL2 | VOL2 |
| 1 | 1 |  |  |  |  |  |

Note before ‘1’ ‘1’ case:

Notice eno shart ykoon el 2 transistors da5el 3leehom nafs el voltage.

‘1’ ‘1’:

By design, M1 and M2 fel ohmic 3ady. Vout and IL? KCL at the output, bs abl ma te3mel keda esta5dem el rule bta3t 2 parallel transistors el lesa 2ayleen 3leeha:

IL=IM,parallel

KL2VDD-VOL3-VTHL2=KM,parallelVDD-VTHL-VTHM1,2VOL3-VOL322

KM,parallel=K1+K2

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Cut-off | Cut-off | Sat. | 0 | VDD-VTHL |
| 0 | 1 | Cut-off | Ohmic | Sat. | IOL1 | VOL1 |
| 1 | 0 | Ohmic | Cut-off | Sat. | IOL2 | VOL2 |
| 1 | 1 | Ohmic | Ohmic | Sat. | IOL3 | VOL3 |

Power Dissipation?

P=(0+IOL1+IOL2+IOL3)4*VDD

What is the relation between VOL1, VOL2 and VOL3?

VOL3<VOL1,VOL2

Again, fakar fiha ka voltage divider. Fel case beta3t ‘1’ w ‘1’, bema en M1 & M2 became on, fa ka2en ba2a fih 2 resistors in parallel, w bel taly el value beta3et-hom 2alet, w bel taly el voltage drop 3leehom 2al, el howa VOL3.

### NAND Gate

| A | B | M1 | M2 | ML | IL | Vout |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Cut-off | Cut-off | Sat. | 0 | VDD-VTHL |
| 0 | 1 | Cut-off | Ohmic | Sat. | 0 | VDD-VTHL |
| 1 | 0 | Ohmic | Cut-off | Sat. | 0 | VDD-VTHL |
| 1 | 1 | Ohmic | Ohmic | Sat. | IOL | VOL |

IL=IM,series=IOL

KL2VDD-VOL-VTHL2=KM,seriesVDD-VTHL-VTHM1,2VOL-VOL22

KM,series=K1K2K1+K2

Determine the logic expression of the shown circuit:

Answer:

F=AB+CD+E→F=(AB+CD+E)

Draw the circuit that represents the following expression:

F=AB+CDE

Back to this circuit again:

What is the output in this case:

| A | B | C | D | E |
| --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | 1 |

Answer:

Mabda2eyan howa akeed logic ‘0’. Bs kam bzbt? Terza3 el equation el gamela:

KL2VDD-VOL-VTHL2=KM,seriesVDD-VTHL-VTHMA,BVOL-VOL22

KM,series=KAKBKA+KB

What is the output in this case:

| A | B | C | D | E |
| --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 |

Answer:

Mabda2eyan howa akeed logic ‘0’. Bs kam bzbt? Terza3 el equation el gamela:

KL2VDD-VOL-VTHL2=KequivalentVDD-VTHL-VTHMC,D,EVOL-VOL22

Kequivalent=KC(KD+KE)KC+KD+KE

What is the output in this case:

| A | B | C | D | E |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 |

Answer:

Logic ‘1’, which is VDD-VTHL

What is the input that results in the best possible VOL? And what is the value of this VOL?

Answer:

When all inputs are high (Again, fakar fel transistors ka resistors, kol ma el parallel branches ketro kol ma el value 2alet and hence el voltage el 3leehom, el howa VOL, hay2el).

KL2VDD-VOL-VTHL2=KequivalentVDD-VTHL-VTHVOL-VOL22

Kequivalent=KAKBKA+KB+KC(KD+KE)KC+KD+KE