Q: assume two asynchronous domains operating with the same clock frequency. If the transmitter is transmitting all the time, then what type of synchronization would we need here?

A: asynchronous FIFO

Q: in the previous question, what should be the depth of the FIFO?

A: bsbb enena ben synchronize el read pointer lel Tx domain through a double flop synchronizer, and we synchronize the write pointer lel Rx domain through a double flop synchronizer, wel double flop synchronizer el latency bta3to 1-2 clock cycles fihom uncertainty be 1 cycle, fa in the worst case el read pointer haya5od 2 cycles 3o2bal ma ye get synchronized lel Tx domain, wel write pointer haya5od 2 cycles 3o2bal ma ye get synchronized lel Rx domain, fa dool 4 cycles hykoon el Tx katab fihom, fa el FIFO depth should be 4.

Q: ezay ne3raf 3adad el taps bta3t an FIR filter men el poles el fel transfer function bta3to?
A: mesh 3aref el egaba bs mohem gedan gedan a3raf w howa kaman 2aly en mohem in general teb2a 3aref how to design digital filters from scratch.

Q: what is the maximum quantization noise in LSBs of the ‘Nearest’ and ‘Floor’ rounding methods in MATLAB?

A: ‘Nearest’ bet2arab el raqam le a2rab quantization level, fa el worst case en ana ab2a mabeen two quantization levels, fa el error yeb2a half a quantization step, el heya half an LSB. ‘Floor’ bet2arab el raqam le t7t, fa lw raqam orayeb awy men quantization level men t7t, hatroo7 me2arabah le t7t, fa el maximum error howa 1 LSB kamla.

Q: assume we want to multiply 2 3-bit numbers. How many bits would the result maximally need?

A: 6 bits

Q: in the previous question, if we allocated 5 bits only to the result, what would happen?

A: overflow

Q: in the previous question, when exactly would an overflow take place?

A: assuming signed 3-bit numbers with no fractional bits, they take the range [-4, 3]. Thus, their multiplication maximally would produce the range [-12, 16]. The range [-12, 15] requires only 5 bits, while only “16” requires 6 bits. Thus, the only result we would lose in case we allocated the result 5 bits only is when the result is 16. See? Only one case! On the other hand, if we assumed unsigned numbers, they take the range [0, 7], thus, their multiplication maximally would produce the range [0, 49]. The range [0, 31] requires only 5 bits, while the range [32, 49] requires 6 bits. Thus, allocating 5 bits only to the result in case of unsigned numbers would lose us larger range than the signed case.

Q: do we need to equalize for the wireless channel in both single-carrier and multi-carrier systems or one of them only?

A: both

Q: in the previous question, if both systems require equalization, then what is the advantage of using multi-carrier system to a single-carrier system from equalization point of view?

A: a multi-carrier system doesn’t omit the need for an equalizer, but it simplifies it drastically thanks to the idea of dividing the channel into subcarriers, hence the equalization becomes a simple complex multiplication per subcarrier. Need to read more about channel equalization in single-carrier and multi-carrier systems.